TrueMotion(R) S for Sega Saturn / Hitachi SH2
Developers' Reference
Preliminary Version TM 102
Dec 1994

Copyright 1994-95 The Duck Corporation

Contents

µIntroduction	3

The TrueMotion(R) S Player	3

Memory Requirements	3

DKAVIFilePlay	4

DKAVIFileOpen	5

DKAVIFileClose	6

getNextFrame	7

getNextAudio	8

releaseChunk	9

fillBuffer	10

drawFrame	11

decodeFrame	12

setFrameBuffer	13

freeFrameBuffer	14

getStereoSample	15

getMonoSample	16

## Introduction

The TrueMotion(R) developer's libraries consist primarily of 2 library files (libduk.and libavi.a for use with the GNU tools and sega_avi.lib and sega_duk.lib for the Hitachi Tools) and a sample TrueMotion(R) Audio/Video player implementation.

Also included are the Audio and Video Coducks (Codecs) for Microsoft Video for Windows as well as tools for their use within Microsoft Video for Windows.

## The TrueMotion(R) S Player

In order for you to include TrueMotion(R) audio and video in your application, we have developed an extensible player shell compatible with both the GNU and Hitach tool sets.  The provided player is sample code and as such will most likely require modification for integration with your application.  In integrating the player with your application please pay close attention to memory managment, and sound and video initialization.  In its native state the player will more than likely happily clobber other code/drivers/memory it is unaware of.

The player's basic architecture relies upon the Saturn's VDP1 for rendering video data to the screen and therefore supports any manipulation you would expect of a VDP1 distortable sprite.  Additionally, other sprites and backgrounds can be combined with the video "sprite".  Be aware however, that there is a limit to what can be done simultaeneously with video and audio decompression.  Please feel free to experiment with the player.  We look forward to seeing the results.

The player sets up three frame buffers in the VDP1's VRAM for decompression in order to allow playback to proceed smoothly during heavy loading of the system.  During the sprite end draw interrupt, the command table of the VDP1 is updated to point to the appropriate frame buffer and any used frame buffer is released.  The main program loop continuously checks to see if there are any buffers available and if so decompresses frames into them.

All timing is derived from the vertical blank interrupt in combination with the PCM play address of the audio subsystem.  By using both these sources, a tight sync can be maintained between audio and video.  It is also within the vertical blank that the audio buffers are loaded.

## Memory Requirements

The TrueMotion(R) S Player compiles to approximately 200K of machine code and statically allocated data (which includes most of the Sega Saturn development libraries).  The AVI and DUK libraries themselves contribute approximately 64K to the size of the player.

The libraries also dynamically allocate 280K for internal structures and an additional 128+K (application specific) CDA/V buffer.

Memory is allocated via the routines found in duckmem.c.  Patch your memory management calls into DUKMEM.C to control it's memory usage via your application.

As written, the player can in under 512K total SH2 memory. (the buffer amount in the inital DKAVIFilePlay call should be changed to 128*1024).  Although using more buffer space for CDA/V is never a bad thing, the player can run well (depending on file datarate) with as little as 128K.

## DKAVIFilePlay

#include<duckfile.h>	Required for function and type declarations;

void DKAVIFilePlay(char *fileName, int bufferSize);

Parameter	Description

fileName	.avi or .duk file to play

bufferSize	size (in bytes) to allocate to A/V CD buffering (passed to DKAVIFileOpen)

The DKAVIFile play function encompasses all the necessary functionality to play a duk or avi file on a specific platform.

Return Value

DKAVIFilePlay does not return a value.

Return Value	Description

none

## DKAVIFileOpen

#include<duckfile.h>	Required for function and type declarations;

int DKAVIFileOpen(
const char *AVIFileName,
DUCK_HANDLE *dckHnd,
int bfrSize,
int preloadAmt,
int startingFrame);

Parameter	Description

AVIFileName	specifies the name of the .avi or .duk file to open and load

dckHnd	pointer to a location in which to store DUCK_HANDLE

bfrSize	specifies movie buffer size in bytes (will be rounded to 2K)

preloadAmt	amount of buffer to preload with file (also specifed in bytes)
-1 indicates default amount (3/4 of buffer size)
-2 indicates fill entire buffer

startingFrame	frame at which to begin playback

The DKAVIFileOpen function opens and loads an AVI or DUK file for playing.  This function allocates necessary buffers and data structures. Be sure to properly initialize any platform sepecific memory managers before using this function.

128K will be substitued for any bfrSize of less than 128K.

preloadAmount can greatly affect the time required between starting the player and the start of playback.  Remember that 2x CD readers generally have a max data rate of 300KBS and a seek time on the order of 300ms.  By keeping your average data rate under 300K, less buffering and preloading will be necessary and files will load and start faster.

Return Value

DKAVIFileOpen returns a non-negative number on success or a negative error code on failure.

Return Value	Description

-1	File not found

-2	Insufficient memory available

-3 or less	invalid file format

## DKAVIFileClose

#include<duckfile.h>	Required for function and type declarations;

void DKAVIFileClose(DUCK_HANDLE dckHnd);

Parameter	Description

duck	handle to an video file stream opened using DKAVIFileOpen

The DKAVIFileClose function is used to close an open video/audio stream and release any buffers belonging to the file (but not the system).

Return Value

DKAVIFileClose does not return a value.

Return Value	Description

none

## getNextFrame

#include<duckfile.h>	Required for function and type declarations;

int GetNextFrame(DUCK_HANDLE duck, void **frame,long *length);

Parameter	Description

duck	handle to a video file stream opened using DKAVIFileOpen

frame	pointer to a location in which to store a pointer to a raw video frame.   Upon return, this pointer will indicate a position within the TrueMotion(R) AVI driver's buffer area.

length	pointer to area in which to store the size of the compressed frame

The getNextFrame function returns a chunk index corresponding to the next available frame in the opened video stream. Every call to getNextFrame MUST be balanced by a call to releaseChunk or buffer space used by the frame will never be released and starvation will occur.

Return Value

The getNextFrame function normally returns a non negative chunk index corresponding to the next available frame in the opened video stream.  In the event of an error an error code is returned as follows

Return Value	Description

-1	last frame has previously been read from the stream

-2	frame is not yet available.  Usually indicates buffer starvation caused by used but unreleased (stale) chunks or insufficient calls to fillBuffer.

## getNextAudio

#include<duckfile.h>	Required for function and type declarations;

int GetNextAudio(DUCK_HANDLE duck, void **audio,long *length);

Parameter	Description

duck	handle to an video file stream opened using DKAVIFileOpen

audio	pointer to a location in which to store a pointer to the next chunk of raw audio.  Upon return, this pointer will indicate a position within the TrueMotion(R) AVI driver's buffer area.

The getNextAudio function returns a chunk index corresponding to the next available audio chunk in the opened video stream. Every call to getNextAudio MUST be balanced by a call to releaseChunk or buffer space used by the audio chunk will never be released and buffer starvation will occur.

Return Value

The getNextAudio function normally returns a non negative chunk index corresponding to the next available audio chunk in the opened video stream.  In the event of an error an error code is returned as follows

Return Value	Description

-1	last audio chunk within the stream has previously been read.

-2	audio is not yet available.  Usually indicates the buffer starvation caused by used but unreleased (stale) chunks or insufficient calls to fillBuffer.

## releaseChunk

#include<duckfile.h>	Required for function and type declarations;

void releaseChunk(DUCK_HANDLE dckHnd,int chunkIndex,int cleanUp);

Parameter	Description

duck	handle to an video file stream opened using DKAVIFileOpen

chunkIndex	chunkIndex returned by getting either a frame or audio chunk

cleanUp	1 to force immediate index cleanup, 0 to allow automatic cleanup

The releaseChunk function invalidates video and/or audio chunks in the AVI driver's movie buffer.  Chunks cannot be reliably re-used after release.  Chunks must be released in order to allow space for new chunks from files in progress.

When a file is entirely pre-loaded and random or backward play is required, do not use the releaseChunk function.

Return Value

releaseChunk does not return a value.

Return Value	Description

none

## fillBuffer

#include<duckfile.h>	Required for function and type declarations;

int fillBuffer(DUCK_HANDLE duck, int maxBytesThisRead,int reserved);

Parameter	Description

duck	handle to an video file stream opened using DKAVIFileOpen

maxBytesThisRead	Maximum Number of bytes to be read from disk on this particular read or 0 to accept computed default.

reserved	reserved for future use, set to 0

The fillBuffer function reads additional data from the CD file into the space invalidated by calls to releaseChunk or any empty buffer space available.  The MaxBytesThisRead parameter can be used to limit the amount read from disk to a specific value.  Specifying 0 as the maxBytesToRead causes fillBuffer to automatically determine the most efficient amount to read based on the frame and data rate of the file, the amount of data in the CD subsystem's hardware buffer as well as other determining factors.

Most applications should rely on the default values computed internally by the AVI library.  In a real-time player, fillBuffer should be called once and only once per frame tick.

Return Value

fillBuffer returns the number of bytes actually read from the source into the movie buffer or a negative error code.

Return Value	Description

0	no space for writing yet available in the buffer.

-1	End of file has been reached.

## dxFrame

#include<duckfile.h>	Required for function and type declarations;

void dxFrame(unsigned char *comp,unsigned char *dest);

Parameter	Description

comp	pointer to compressed data (received using getNextFrame)

dest	destination into which frame is to be decompressed

dxFrame decompresses a frame into the specified location and informs the caller of the width and height of the actual frame.  Before using dxFrame to decompress a frame, be sure to use setPitch to set the pitch of the area into which dxFrame will decompress.

Return Value

dxFrame does not return a value

Return Value	Description

none

## decodeFrame

#include<duckfile.h>	Required for function and type declarations;

void *decodeFrame(void *data, unsigned long *width, unsigned long *height);

Parameter	Description

data	pointer to compressed data (received using getNextFrame)

width	pointer to location into which frame width is placed

height	pointer to location into which frame height is placed

The decodeFrame function is used to get width and height information from a compressed frame without decompressing it.

Return Value

decodeFrame does not return a value

Return Value	Description

none

## setFrameBuffer

#include<duckfile.h>	Required for function and type declarations;

int setFrameBuffer(void *framedata);

Parameter	Description

framedata	pointer to compressed frame (received using getNextFrame)

Once an AVI file has been loaded and an initial frame has been decoded, the setFrameBuffer function is used to allocate the framebuffer used internally by the decompressor.

Return Value

setFrameBuffer returns the number of bytes it allocated (using duck_malloc).

Return Value	Description

>0	number of bytes allocated

## freeFrameBuffer

#include<duckfile.h>	Required for function and type declarations;

void freeFrameBuffer(void);

Parameter	Description

none

Free space allocated using setFrameBuffer.  Use this function after the last call to dxFrame

Return Value

freeFrameBuffer does not return a value.

Return Value	Description

none

## getStereoSample

#include<duckfile.h>	Required for function and type declarations;

int getNextStereoSample(DUCK_HANDLE duck,short *left,short *right,int numberOfSamples);

Parameter	Description

duck	handle to an video file stream opened using DKAVIFileOpen

left	pointer to destination buffer for left channel information (16 bit)

right	pointer to destination buffer for right channel information (16 bit)

numberOfSamples	number of stereo pairs to read

Return Value

getNextStereoSample returns the number of stereo pairs actually read (may be different then the number specified depending on the state of buffering.

Return Value	Description

0 	No samples available

## getMonoSample

#include<duckfile.h>	Required for function and type declarations;

int getNextStereoSample(DUCK_HANDLE duck,short *left,short *right,int numberOfSamples);

Parameter	Description

duck	handle to an video file stream opened using DKAVIFileOpen

dest	pointer to destination buffer

numberOfSamples	number of samples to read

Return Value

getNextMonoSample returns the number of samples actually read (may be different then the number specified depending on the state of buffering.

Return Value	Description

0 	No samples available